- TOWARDS A FLOOD ALERT SYSTEM FOR EUROPEAN TRANS-NATIONAL

- RIVER BASINS

- K. Wachter1,2, M. Kalas1,3, J. Szabo1,4, J. Younis1,5, S. Niemeyer1, M. Gierk1,6, A. de Roo1

- 1 Institute for Environment and Sustainability, DG Joint Research Centre,

- European Commission, TP 261, 21020 Ispra, Italy

- 2 Detached from Department of Water Management, Government of Lower-Austria, Austria

- 3 Detached from Department of Land and Water Resources Management, Faculty of Civil

- Engineering, Slovak University of Technology, Slovakia

- 4 Detached from Department of Sanitary and Environmental Engineering, University of

- Technology and Economics. Budapest, Hungary

- 5 Detached from Czech Hydrometeorological Institute, Prague, Czech Republic

- 6 Detached from Department of Protection of Waters and Water Resources Management,

- Brandenburg State Environmental Agency, Potsdam, Germany

- corresponding authors: stefan.niemeyer@jrc.it, ad.de-roo@jrc.it

- Abstract: The European Flood Alert System (EFAS) for medium-range flood forecasting in

- trans-national European catchment areas is presented in its concept and its current state of

- development. The flood simulation model is based on the LISFLOOD hydrological model

- already in use for the rivers Odra, Meuse and Elbe. Furthermore, an integrated modelling

- system for the entire Danube catchment will allow for the evaluation of flood protection

- measures and land use changes at the scale of the entire river basin. The contribution will

- present the necessary steps in data collection, processing and the preparation for model

- validation as well as first preliminary modelling results.

- Keywords: Floods, Trans-national River Modelling, Flood Alert System, LISFLOOD,

- Modelling

- EIN EUROPÄISCHES HOCHWASSER-FRÜHWARNSYSTEM FÜR

- GRENZÜBERSCHREITENDE FLUSSGEBIETE

- Zusammenfassung: Dieser Beitrag stellt das Konzept und den aktuellen Stand des

- Europäischen Hochwasser-Frühwarnsystems (EFAS) vor. Das Modell zur europaweiten

- Hochwasservorhersage basiert auf dem hydrologischen Modell LISFLOOD, das schon für

- die Einzugsgebiete der Oder, Meuse und Elbe erfolgreich verwendet wurde. Zusätzlich zur

- Hochwasser-Frühwarnung wird das resultierende integrierte Modellsystem zukünftig auch

- zur Szenariomodellierung verwendet werden, um die Auswirkungen von

- Hochwasserschutzmaßnahmen oder Landnutzungsänderungen abschätzen zu können.

- Neben den notwendigen Schritten der Datenaufbereitung werden erste Ergebnisse der

- Modellierungen in ausgewählten Einzugsgebieten vorgestellt.

- Schlüsselworte: Hochwasser, grenzüberschreitende Flussgebiete, Frühwarnsystem,

- LISFLOOD, Modellierung

- 1. Introduction

- In the last decade, several large floods have occurred in European trans-national

- catchment areas, such as, among many others, the Rhine flooding in the winters of 1995,

- 2001, 2003, or the Odra flooding in 1997 and 2001, the Tisza flood of 2000, or the Danube

- and Elbe flooding in August 2002. The simulation and forecasting of such events, as well as

- scenario modelling for future water management and mitigation measures, requires a

- catchment-wide, trans-national approach. Following the disastrous floods in August 2002 in

- the Elbe and Danube river basins, the European Commission decided to support the

- development of a European Flood Alert System (COM(2002)481-final)). Following this

- decision the Joint Research Centre is developing and pre-operationally testing a European

- Flood Alert System (EFAS), with the aim to provide National Flood Forecasting Agencies

- with additional medium-range information besides their own – typically short-range -

- forecasting systems. As main pilot basins the Danube and the Elbe have been selected, for

- which also flood prevention scenario studies are foreseen. This is achieved in close

- collaboration with national and regional water authorities of the respective member states

- and with support of International River Commissions like for the Danube (ICPDR), the Odra

- (IKSO) and the Elbe River (IKSE). Following the Danube and Elbe pilot studies, other transnational

- river basins will be tested in more detail.

- EFAS will act complementary to local and national forecasting systems and focus on

- medium-range flood forecasts of three to ten days, thus extending the lead time compared to

- pure hydrological forecasting and allowing regional and local authorities to take the

- appropriate measures in case of a flood event. Besides the flood alerting capability of the

- integrated modelling system, it will allow for scenario simulations to evaluate flood protection

- measures and land use changes at the scale of the entire river basin.

- Including the Danube River catchment area with more than 800’000 km² as the

- largest European catchment area with eighteen countries involved into EFAS poses an

- extraordinary effort on data requirements, management, and processing. Regarding the Elbe

- River with a catchment area of about 148’000 km2 three countries and several national and

- regional authorities are involved.

- The contribution presents the current state of development of EFAS with emphasis on

- the Danube and Elbe catchment area, explains necessary steps in data processing and

- preparation for the modelling, and shows first preliminary modelling results. Finally it gives an

- outlook to the next steps to be taken and still pending problems to be overcome in order to

- create an operational European Flood Alert System.

- 2. The European Flood Alert System

- Floods in large river basins are predominantly the consequence of extreme mesoscale

- precipitation events in Europe that usually show a persistence of several days to

- weeks. The European Flood Alert System makes use of these characteristics and combines

- medium-range weather forecasts, e.g. from the European Centre for Medium-Range

- Weather Forecasts (ECMWF) or the German Weather Service (DWD) with forecast times of

- up to ten days, with a hydrological model that comprises the trans-national catchment areas

- throughout Europe. The spatial resolution of the hydrological model initially has been set to

- 5 km and is currently increased to 1 km for all major river basins, including the entire Danube

- catchment area. EFAS is run twice a day corresponding to the availability of meteorological

- forecasts. The underlying hydrological model is the LISFLOOD model (De Roo et al., 2000)

- and will be presented in the following section. Figure 1 shows the principle elements of EFAS

- in its current phase, including ongoing developments.

- After reception of the current weather forecast, the meteorological parameters are

- downscaled to the hydrological model’s spatial resolution. The LISFLOOD model is run,

- making use of previously derived calibration parameters and a set of static data that

- characterize the physical properties of the catchment area. The model run produces

- forecasted discharges on the basis of the forecasted precipitation and the initial conditions of

- the previous model run. In the next step, the produced discharge values are compared to

- long-term statistical threshold values derived from water balance runs over several decades.

- Currently, forecasts are compared to water balance runs since 1990. As a next step, the

- ECMWF ERA40 data will be used to obtain a 44-year discharge climatology. These water

- balance runs have been performed beforehand, making use of the same model code to be

- applied in the flood forecasting mode, as well as of calibration data derived from model runs

- with long-term local precipitation and measured discharge data. If the simulated discharge

- exceeds chosen threshold values of the water balance runs consistently for consecutive

- forecasts, it will be taken as a strong indication of a flood event to be forecasted in the

- respective region, and a flood alert will eventually be issued to the respective national or

- regional river water authority.

- Figure 1: Overview sketch of the current EFAS system,

- including ongoing developments (shaded boxes).

- At the same time, after reception of measured data like precipitation or temperature

- from synoptic stations, river discharges from gauging stations, or products derived from

- remote sensing, these data are assimilated into the modelling system in order to update the

- forecasts by the most accurate, measured values available.

- As a novelty in hydrological flood simulation, EFAS explicitly will take into account the

- uncertainty of medium-range weather forecasts, since the accuracy of the forecasted

- parameters, especially the absolute quantity of precipitation, is decreasing considerably with

- increasing lead time. EFAS will incorporate so-called ensemble predictions consisting of a

- set of several tens of meteorological forecasts computed from slightly different initial

- conditions and different model parameterisations. Using an ensemble of different realisations

- of meteorological forecasts as the input rather than relying on a single meteorological

- forecast, several runs of the hydrological model will produce an ensemble of discharge

- forecasts as well, thus allowing for the quantification of the uncertainty of the forecasted

- discharge.

- Besides producing medium-range flood forecasts by EFAS, the modelling system is

- used in parallel for scenario modelling in different catchment areas throughout Europe. In this

- approach, the consequences of various potential flood protection measures can be simulated

- for the large catchment areas, benefiting from the trans-national model set-up for EFAS. At

- the moment, a study is ongoing in the framework of the Elbe Flood Action Plan from the

- IKSE on the effect of reservoirs and polders on the Elbe discharge.

- 2.1. The LISFLOOD model

- The physically based LISFLOOD (De Roo, 1999, De Roo et al., 2000) model has

- been developed explicitly for the simulation of floods in large European drainage basins. It is

- capable of simulating large areas, while still maintaining a high resolution, proper flood

- routing methods and overall physical process descriptions. LISFLOOD is embedded in the

- PCRaster GIS (Wesseling et al., 1995) and is using readily available European datasets,

- such as Corine Land Cover (EC, 1993) and the European Soils Database (King et al., 1995

- and Heineke et al. 1998). LISFLOOD simulates the hydrological processes at the surface, in

- the soil, and in the river channel network on a regular horizontal grid (figure 2).

- In the ground a total of four different layers are considered. For each grid point a

- value is calculated at every time step. Processes simulated are interception, soil freezing,

- snowmelt, evapotranspiration, infiltration, percolation and capillary rise, groundwater flow and

- surface runoff.

- Overland flow is simulated using a kinematic wave approximation. Channel flow is

- simulated using either a kinematic wave or dynamic wave approximation, depending on river

- channel bed gradient and the occurrence of backwater effects. The cross section of the river

- and associated floodplain is taken into account by using series of water-level, wetted

- perimeter, and hydraulic radius values for locations for which river geometry is available. The

- user can define which sections of the river to simulate with a kinematic wave, and which

- sections with a dynamic wave. The user also can choose both the spatial and temporal

- resolution of the model. For European trans-boundary flood simulations, typically a grid-size

- of 1 km and a time-step of 1 h are used. A detailed model description can be found in De

- Roo et al. (2000).

- Figure 2: Schematic view of a catchment in LISFLOOD

- including soil and groundwater layers.

- The input parameters for the LISFLOOD catchment model are maps of topography,

- land use, soil depth, and soil texture (see figure 3). Time series of precipitation amounts and

- other meteorological parameters (minimum and maximum daily air temperature, actual

- vapour pressure, sunshine duration, cloud cover, horizontal wind velocity) are needed for as

- many meteorological stations within the catchment as possible. All meteorological

- parameters are spatially interpolated, if necessary, and where appropriate the variables are

- corrected for altitude, like e.g. air temperature.

- The output of LISFLOOD consists of hydrographs at user-defined locations within the

- catchment area, usually at the locations of gauging stations where the measured discharge

- is known, too. Furthermore, time series of model parameters like, e.g. evapotranspiration,

- soil moisture content or snow depth, can be created at selected locations. The model can

- produce maps of any simulated variable, such as water source areas, discharge coefficient,

- total precipitation, total evapotranspiration, total groundwater recharge or soil moisture.

- Figure 3: Data requirements of the LISFLOOD model.

- 3. Data preparation and model set-up

- Data collection and preparation for the model runs requires substantial efforts and a

- chain of tasks that consist of the following steps:

- • data collection

- • integrity checking of incoming data

- • data conversion: re-formatting of received heterogeneously organised data

- • transformation of co-ordinates and altitude to common reference system

- • detection and correction of outliers in meteorological data

- • spatial interpolation of meteorological data

- • conversion of cross section geometry into channel properties for model input

- • implementation of reservoirs and polders into the model

- • model validation: comparison and evaluation of simulated and observed

- discharges

- Of this list, data collection, co-ordinates’ transformation and spatial interpolation will

- be discussed in more detail in this chapter.

- Furthermore, the large trans-national European catchment areas have been divided

- into sub-catchments during data preparation, in order to work in parallel on different subcatchments,

- in correspondence to the continuously incoming data, as well as to be able to

- calibrate sub-catchments by using local data in high spatial resolution. As an example, data

- processing has already been started or even completed on parts of Morava River, most

- Slovakian rivers, the Tisza and Sava catchment areas. While the Elbe River catchment area

- has been divided into nine sub-catchments, the Danube River catchment area has been

- divided into the eight sub-catchments Upper Danube until Achleiten, the Danube stretch

- down to Bratislava, the Morava catchments including Vah and Hron, Drava, Tisza, Sava, the

- Danube down to the Iron Gate, and the Lower Danube (see figure 4).

- Figure 4: The Danube catchment area divided into eight sub-catchments.

- 3.1. Data collection

- The Danube catchment area comprises eighteen countries, of which fourteen

- covering the major part of the basin have been contacted for data collection. Relevant

- authorities that hold necessary data are usually the national meteorological and hydrological

- services, administrations responsible for landuse, soil and geological data as well as

- authorities of national and regional water resources management. Some of these branches

- are divided again into federal and regional or even local administrations with different tasks

- so that sometimes it has been difficult to get in contact with the actually responsible person.

- These problems have been encountered especially in countries with a distinct federal

- administrative structure. Consequently it has been necessary to get in touch with more than

- 40 administrations altogether. Looking at data collection for the German part of the Elbe

- catchment area, for example, at least six German state authorities and two federal institutes

- have had to be contacted. For one of the biggest tributary only, the Saale, four German

- states and one federal institute are responsible. On the contrary, the Czech part of the Elbe

- catchment area is represented by three institutes only.

- In addition, experience has shown that data delivered are usually very different in

- format and content of the respective files; some data are not available in digital format at all.

- 3.2. Data transformation

- Information on discharge gauging stations, rainfall stations, cross sections and

- reservoirs as well as polders usually arrives with co-ordinates according to each country’s

- standards. The definition of the corresponding projection systems and the precision of coordinates

- vary considerably among the countries, and often even within one country or within

- one authority (e.g. different dates of measurement, methodologies, geodetic surveys). Hence

- it is necessary to convert these data to a common altitude and co-ordinate reference system

- before feeding them into the LISFLOOD model. Regarding altitude, the European Altitude

- reference system has been chosen (Altitudes of Amsterdam), while for the horizontal

- projection the Lambert Azimuthal Equal Area reference system has been adopted (ETRS89-

- LAEA, Annoni et al., 2003).

- In order to perform the projection transformations, all necessary projective

- parameters of the source projection system have to be known. Unfortunately, this piece of

- information is often not provided together with the data delivery, sometimes it is not known to

- the data providers, either. In these cases the projection parameters have to be examined and

- retrieved on a case by case basis.

  - Processing of precipitation data

- As precipitation is the most important input to a hydrological model, the precipitation

- data received are processed carefully before being submitted to the modelling system.

- Basically there are two sources of precipitation data. On the one hand, at JRC a

- meteorological data base called MARS data base is existing (EC, 1998) that holds daily point

- data from synoptic stations and spatial fields of meteorological parameters. On the other

- hand, high-resolution historical measured rainfall data have been received from the

- respective national, regional and local authorities. Besides precipitation, both sources of data

- usually provide air temperature and sometimes the parameters horizontal wind speed, air

- humidity and/or solar radiation input as well.

- Before a spatial interpolation of the station data can be performed, a filtering of the

- time series have had to be performed in order to identify outliers and anomalies in the

- received precipitation data. A study on spatial interpolation method showed that Kriging using

- an adaptive recognition algorithm of local anisotropy is the most suitable method for

- interpolating point data of precipitation (Szabó, 2004). Figure 5 shows an example of the

- chosen interpolation method in comparison to the MARS data.

- Figure 5: Example of interpolated precipitation data; left: MARS data, right: local network

- data interpolated with Kriging using adaptive recognition of local anisotropy

- (17.06.1997, Morava catchment area).

- The influence of the meteorological input data on the resulting hydrograph is illustrated in figure 6. Because the MARS data are based on the synoptic network whose

- density is lower than national networks, the volume of interpolated precipitation is generally

- underestimated, producing a generally lower discharge values compared to the hydrograph

- from high-resolution station data.

- Figure 6: Comparison between the resultant simulated hydrographs of Morava River

- at gauging station Kromerir, 1998, using MARS (Mars) and local network data (NN).

- 4. First results of simulation

- The first simulations with LISFLOOD were run using both MARS and high resolution

- historical meteorological data received from the national authorities. Figure 7 shows first

- results for the Elbe River catchment area with MARS data; it produces big differences

- between simulated and observed discharge data. Figure 8 shows the same simulation with

- high resolution meteorological data; the resulting discharge values are in better

- correspondence with the measured discharges.

- Figure 7: Simulated and measured discharge of Elbe River at Brandys nad Labem gauging

- station, Poland, 1.10.1998 – 30.09.1999, using MARS rainfall data.

- Figure 8: Simulated and measured discharge of Elbe River at Brandys nad Labem gauging

- station, Poland, 1.10.1998 – 30.09.1999, using high resolution historical meteorological data.

- Figures 9 and 10 show simulation results for the Morava River at Kromerir gauging

- station and for Hron River at Banska Bystica gauging station, both within the Danube River

- catchment area. For both simulations, high resolution meteorological data have been used.

- Measured discharge values are reasonably well reproduced by the model.

- Figure 9: Simulated and measured discharge of Morava River at Kromerir, Slovakia,

- using high resolution meteorological data.

- Figure 10: Simulated and measured discharge of Hron River at Banska Bystica, Slovakia,

- using high resolution meteorological data.

- First interim modelling results have been obtained from for the Upper Danube

- catchment area as well (see figure 11).

- Figure 11: Preliminary LISFLOOD simulation result of the discharge of the Upper Danube at

- Achleiten, Austria (see arrow) for 2000 to 2002. MARS data has been used for

- meteorological input data and no calibration has been performed so far.

- As local precipitation data have not been available yet, the simulations have been run

- with MARS data. No calibration of the hydrological model has been performed so far. While

- the general annual variation and especially the flood in August 2002 has been well

- reproduced, minor peaks especially between summer 2001 and 2002 have been

- underestimated by the simulation, which might be caused by the already mentioned

- underestimation of the precipitation data. Further improvement and calibration is ongoing.

- 5. Conclusions

- Setting-up the LISFLOOD model for the European Flood Alert System and scenario

- modelling in trans-national catchment areas comprises many steps from data collection to

- model calibration. The choice of the precipitation data input has proved to be most critical to

- the resulting hydrographs. The results presented in this work show the usefulness of having

- high-resolution rainfall and temperature data as input to the model. This is especially true for

- regional scenario modelling, while for European-wide flood forecasts the model will have to

- be run with the existing spatial forecast data. For the updating procedure of the modelling

- system, the inclusion of additional precipitation data from automated national or regional

- networks would be of great advantage.

- 6. References

- Annoni, A., Luzet, C., Gubler, E., and Ihde, J. (eds.) (2003): Map projections for Europe.

- Institute for Environment and Sustainability, Joint Research Centre, Commission of

- the European Communities, Ispra, Italy.

- De Roo, A.P.J. (1999): LISFLOOD: a rainfall-runoff model for large river basins to assess the

- influence of land use changes on flood risk. In: Balabanis, P. et al. (eds.): Ribamod:

- river basin modelling, management and flood mitigation. Concerted action, European

- Commission, EUR 18287 EN, 349-357.

- De Roo, A.P.J., Wesseling, C.G., Van Deursen, W.P.A. (2000): Physically-based river basin

- modelling within a GIS: The LISFLOOD model. Hydrological Processes, 14, 1981-

- 1992.

- EC (1993): CORINE Land Cover – Guide technique. Commission of the European

- Communities, Office for Official Publications of the European Communities, EUR

- 12585 FR, Luxembourg, 144 pp.

- EC (1998): Agrometeorological applications for regional crop monitoring and production

- assessment. D. Rijks, J.M. Terres, P. Vossen (eds.), Joint Research Centre of the

- European Commission, Ispra, Italy, EUR 17735 EN, 31-55.

- Heineke, H.J., Eeckelmann, W., Thomasson, A.J., Jones, R.J.A., Montanarella, L. and

- Buckley, B. (eds.) (1998): Land Information Systems: Developments for planning the

- sustainable use of land resources. European Soil Bureau Research Report No. 4,

- Office for Official Publications of the European Communities, EUR 17729 EN,

- Luxembourg, 546 pp.

- King, D., Jones, R.J.A. and Thomasson, A.J. (1995). European Land Information Systems for

- Agro-environmental Monitoring. Office for Official Publications of the European

- Communities, EUR 16232 EN, Luxembourg, 285pp.

- Szabó, J. A. (2004): Application of Adaptive Recognition Algorithm of Local Anisotropy in

- Case of Spatial Interpolation of Precipitation Fields with Kriging. In: Proc. XXIInd

- Conference of the Danubian Countries on the Hydrological Forecasting and

- Hydrological Bases of Water Management. 30 Aug. – 02 Sept. 2004, Brno, Czech

- Republic (to appear).

- Wesseling, C.G., D. Karssenberg, W.P.A. Van Deursen & P.A. Burrough (1995): Integrating

- dynamic environmental models in GIS: The development of a dynamic modelling

- language. Transactions in GIS, 1, 40-48.