FORECASTING RELATIONS OF THE FLASH FLOODS MAXIMUM DISCHARGES ON THE BASIS OF THE CORRELATION WITH THE RELEASING FACTORS

Rodica Mic, Ciprian Corbuş

National Institute of Hydrology and Water Management, Bucureşti, Romania,

e-mail: rodica.mic@hidro.ro; corbus@hidro.ro

Abstract: The lag-time to the rainfall, in case of small areas basins (S<100200km2) is from about 15 minutes to a few hours. Therefore, the afferent time for the flash floods forecasting is very short, so that this activity should be concentrated especially on the danger warning system.

In this situation, in order to assure real time warnings, it is not recommended to use the hydrological modelling; on the contrary it is preferable to rapidly assess the precipitation values supplied by the automatic stations, corroborated with the radar information and the comparing of these precipitation values with different critical thresholds of the precipitation associated to flooding level, pre-established.

The main purpose of this work is to increase the warning time in case of flash floods in torrential regime basins, by elaborating a simple methodology of inundation warnings based on the correlation between the peak discharges of the flash floods with causal factors.

The elements that stand on the basis of the maximum discharge forecast relations are: the precipitation, the duration of the rain, the soil previous humidity and the base discharge.

Keywords: flash floods, hydrological forecast

Scheitelabflüsse PROGNOSE RELATIONEN VON DER STURZFLUTEN AUFGRUND DER MIT DEN AUSLÖSENDEN FAKTOREN KORRELATION

Kurzfassung: Die Verzögerungszeit des Regens, im Falle der kleinen Zuflussgebiete (S<100200km2), variiert von 15 Minuten bis einige Stunden. Folglich, die notwendige Zeit der Sturzflutensprognose ist sehr kurz und darum muss sich diese Aktiwität besonders auf dem Warnungssystem der von den Sturzfluten representiert Gefahr konzentrieren.

Wegen dieser Situation, für rechtzeitige Warnungenssicherungen, empfielt es sich, nicht Flussmodell zu benutzen, sondern ist es vorzuziehen, eine schnelle Bewertung der Niederschlägeswerte, von den automatischen Stationen geliefert, mit der Radarsinformation korreliert und diese Werte mit den Werten der verschiedenen kritischen Niederschlägsschwelle, mit früheren festgelegten Schutzwassersspiegel zuziehend, gleichzusetzen.

Der Arbeitshauptzweck ist die Vergrösserung der Antizipationszeit, im Falle der Sturzfluten, auf Einzugsgebiete mit strömenden Regime, durch die Ausarbeitung einer einfache Methodologie für die Ausgabe der Überflutungenswarnungen, die auf der Scheitelabflüsseskorrelation der Sturzfluten mit den auslösenden Faktoren beruht.

Die Grundeselemente der Prognosesrelationen sind: die Niederschläge, die Regenzeit, die vorherige Grundesfeutigkeit und den Basisabfluss.

Schlüsselworte: Sturzfluten, hydrologische Prognose.

Introduction

The flash floods represent one of the most frequent natural disasters in many countries.

These floods have an extremely rapid evolution and constitute an unexpected element for the inhabitants of the flooded area. In addition, the flooding produced by these floods is difficult to be evaluated, because of the space and time variability of the rainstorm.

From the operational forecast point of view the flash floods are characterized by a very short lag time. Moreover, in case of flash floods, the forecasting activity is concentrated especially on the danger warning system.

A hydrological forecast in real time supposes both the cognition of the hydro-meteorological situation from the basin as exactly as possible and the existence of some rapid methodologies for estimating the maximum discharges at the reaching of some precipitation thresholds (Georgakakos, 1996).

1. Utilized data

For this study were chosen 8 sub-basins (figure 1), from the drainage basin of Crisuri, with a rapid flow regime and a small basin surface (S<100200km2).

#### Figure 1. Drainage basin of Crişuri

#### Selected gauging stations and meteorological stations

In table 1 are presented for the selected gauging stations, the surface and the mean altitude of the basins, and also the mean increase time of the floods.

There were also selected some meteorological stations situated on the drainage basins or close to them and considered to be representative from precipitation on basin point of view.

#### Table 1. Selected hydrometrical stations

| River – Gauging station | Code | S (km2) | Hm (m) | tc (h) |
| --- | --- | --- | --- | --- |
| ## V. de Lazuri - VÂRFURILE | 30805 | 78.6 | 648 | 15 |
| Sighişoara - BRAZI | 30905 | 120 | 455 | 6 |
| ## Sebiş - SEBIŞ | 31205 | 209 | 432 | 16 |
| V. Roşie - POCOLA | 33710 | 267 | 427 | 17 |
| V. Nouă - HUSASĂU DE TINCA | 34005 | 136 | 217 | 16 |
| Henţ - MORLACA HENŢ | 36305 | 209 | 1014 | 20 |
| Iad - LEŞU AMONTE | 36615 | 51 | 1088 | 12 |
| Borod - TOPA de CRIS | 37005 | 115.5 | 484 | 6 |

# 1.1. Rain-flow events selection

##### In a first stage there were chosen all floods whose maximum discharges has values over or close to the discharge corresponding to the alarm level.

If, after this first criterion, the events are much fewer, there were also selected floods with lower maximum discharges, so that for each section are analysed about 20-30 rainfall-runoff events.

In order to determine as exactly as possible the runoff volume, when the rain-flow events were selected, it was also taken into consideration the fact that the selected floods must be complete from hydrometrical point of view.

To assess the duration of the precipitation, another criterion taken into consideration was that the selected floods were produced during the functioning of the recording rain gage at the meteorological stations.

Table 2 presents the total number of analysed events and also the number of events used for the calibration and validation of the computed relationships of the maximum discharges.

#### Table 2. Number of the analysed rain-flow events

| River – Gauging station | Code | S (km2) | No. of events |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Total | Calibration | Validation |
| V. Satului - BUCEŞ | 30205 | 100 | 12 | 8 | 4 |
| ## V. de Lazuri - VÂRFURILE | 30805 | 78.6 | 21 | 15 | 6 |
| Sighişoara - BRAZI | 30905 | 120 | 23 | 16 | 7 |
| ## Sebiş - SEBIŞ | 31205 | 209 | 33 | 15 | 8 |
| V. Roşie - POCOLA | 33710 | 267 | 17 | 12 | 5 |
| V. Nouă - HUSASĂU DE TINCA | 34005 | 136 | 20 | 14 | 6 |
| Henţ - MORLACA HENŢ | 36305 | 209 | 27 | 18 | 9 |
| Iad - LEŞU AMONTE | 36615 | 51 | 20 | 14 | 6 |
| Borod - TOPA de CRIS | 37005 | 115.5 | 21 | 14 | 7 |

1.2. Elements used in the estimated relations of the maximum discharge

The elements that are considered for the maximum discharge forecasting relations of the flood Qmax (m3/s) are:

- Precipitation depth, hp (mm);

- Rain duration, Dp (h);

- Antecedent soil moisture, Ui (mm);

- Base runoff, Qb (m3/s).

The precipitated depth hp (mm) was considered for one or more meteorological stations, considered to be representative for each analysed basin.

The precipitation duration Dp (h), which releases the flood, was estimated after the simultaneous analysis of flood and hourly distribution of the rain registered to the meteorological station, considered being significant for each analysed basin.

The antecedent soil moisture Ui (mm), namely the soil humidity at the moment of the flood release, was computed by a balance method, which estimates that the water quantity remained in the soil during the given time period is the difference between: the quantity of mean precipitation on basin, depth of runoff and evapotranspiration.

Computing formula of the antecedent soil moisture (Simota & Mic, 1993) is the following:

Ui=(1−α)⋅i=110Pi−N⋅E					(1)

in which: α - the flow coefficient; E  - the evapotranspiration; Pi - the daily mean precipitation on basin; N - the number of days without precipitation.

The α coefficient is considered being a function of Pi, and E depends on the month when the flood occurs.

2. Analysis of the link between the maximum discharge and the determinant characteristic elements

On the basis of the selected data, for each of the 8 hydrometrical stations was analysed the dependence of the maximum discharges of the flood waves (Qt), on different characteristics of the generating rain, antecedent soil moisture and base runoff and which one of these characteristics has the biggest influence upon the maximum discharge.

A measure of the "goodness of fit" is the correlation coefficient. To explain the meaning of this measure, it is define the standard deviation (St), which quantifies the spread of the data around the mean (Dauphin, 1992):

St=i=1Ny−yi2					(2)

where the average of the data points () is simply given by :

y=1Ni=1Nyi						(3)

The quantity St considers the spread around a constant line (the mean) as opposed to the spread around the regression model. This is the uncertainty of the dependent variable prior to regression. It is also define the deviation from the fitting curve as:

Sr=i=1Nyi−f(xi)2					(4)

This quantity likewise measures the spread of the points around the fitting function. Thus, the improvement (or error reduction) due to describing the data in terms of a regression model can be quantified by subtracting the two quantities. Because the magnitude of the quantity is dependent on the scale of the data, this difference is normalized to yield:

r≡St−SrSt						(5)

where: r is defined as the correlation coefficient.

As the regression model better describes the data, the correlation coefficient will approach unity.

By the partial correlations computation it has been ascertained that, in the most cases, the maximum discharge of the flood waves does not depend only on a single considered, but on two or more (table 3).

Table 3. Dependence of the maximum discharge Qt on different characteristics of the generating rain (Hp, Dp), antecedent soil moisture (Pa) and base runoff (Qb)

| Independ. variable | Computed formula | r |
| --- | --- | --- |
| Valea de Lazuri - VÂRFURILE |  |  |
| Hp | Qt = 1 / ( 0.20081999 - 0.03994878 * ln(Hp)) | 0.831 |
| Dp | Qt = 16.035533 (1 - exp(-0.35520912 Dp) | 0.537 |
| Pa | Qt = 11.623875 + 0.15781238 Pa | 0.192 |
| Qb | Qt = 14.470705 * Qb(0.31548859 / Qb) | 0.402 |
| Sighişoara - BRAZI |  |  |
| Hp | Qt = 19.829622 * 1.0184108Hp | 0.287 |
| Dp | Qt = 7.9322475 * Dp(6.5661409 / Dp) | 0.426 |
| Pa | Qt = 1 / (-0.0023043848 Pa + 0.08395441) | 0.886 |
| Qb | Qt = 345.08176 * exp((-(3.8208357 - Qb)2) / (2 * 0.687353982)) | 0.865 |
| Sebiş - SEBIŞ |  |  |
| Hp | Qt = 15.760759 + 0.98242748 Hp | 0.832 |
| Dp | Qt = 27.606309 + 1.4790027 Dp | 0.575 |
| Pa | Qt = 40.514192 + 34.266878 * cos(0.96753018 Pa + -1.6473246) | 0.726 |
| Qb | Qt = 58.15321 / (1 + 58.15321 * exp(-3.0520454 Qb)) | 0.542 |
| Valea Roşie - POCOLA |  |  |
| Hp | Qt = -0.36216661 (-133.45852 - exp(0.084141265 Hp)) | 0.817 |
| Dp | Qt = 1 / (0.024795616 - 0.0040890931 * ln(Dp)) | 0.250 |
| Pa | Qt = (58.431333 - 0.72767901 Pa) / (1 - 0.0095076735 Pa -0.0004077003 Pa2) | 0.884 |
| Qb | Qt = -110.68565 + 118.85963 Qb - 25.613628 Qb2 + 1.778826 Qb3 | 0.911 |
| Valea Nouă - HUSASĂU DE TINCA |  |  |
| Hp | Qt = -26.330923 + 2.7997453 Hp - 0.04625257 Hp2 + 0.00027075744 Hp3 | 0.963 |
| Dp | Qt = (-9.3474119 + 3.7717792 Dp) / (1 - 0.31962447 Dp + 0.030853629 Dp2) | 0.522 |
| Pa | Qt = 41.199927 (1 - exp(-0.060052158 Pa) | 0.302 |
| Qb | Qt = 24.231073 + 23.473742 * cos(3.3710656 Qb + 2.5396367) | 0.581 |
| Henţ - MORLACA HENŢ |  |  |
| Hp | Qt = 15.841018 + 0.40476923 Hp | 0.452 |
| Dp | Qt = 35.223221 - 0.38839101 Dp | 0.263 |
| Pa | Qt = 18.33618 + 0.81163033 Pa | 0.489 |
| Qb | Qt = (19.226409 - 2.3489978 Qb) / (1 - 0.17812628 Qb + 0.006727063 Qb2) | 0.673 |
| Iad - LEŞU AMONTE |  |  |
| Hp | Qt = 6.5574791 + 0.39856892 Hp | 0.899 |
| Dp | Qt = (14.789763 - 0.44716952 Dp) / (1 - 0.063804084 Dp + 0.0011103085 Dp2) | 0.524 |
| Pa | Qt = 43.609575 - 6.283673 Pa + 0.41637128 Pa2 - 0.0078025015 Pa3 | 0.759 |
| Qb | Qt = 23.414143 + 7.0990081 * cos(13.894476 Qb - 9.7706268) | 0.444 |
| Borod - TOPA de CRIS |  |  |
| Hp | Qt = 92.125577 * exp((-(75.834427 - Hp)2)/(2 * 30.8266012)) | 0.780 |
| Dp | Qt = 44.810415 - 3.2527198 Dp + 0.14934257 Dp2 | 0.535 |
| Pa | Qt = 47.679318 - 1.3626913 Pa - 0.18866449 Pa2 + 0.0093902641 Pa3 | 0.693 |
| Qb | Qt = 24.948018 Qb ( 0.44379541 Qb) | 0.742 |

That is why it was tried to be used the linear multiple regressions for estimating the maximum discharge depending on the characteristics of the rain (hp, Dp) and on the initial state of the basin (Pa, Qb).

3. Computation of the maximum discharge by multiple linear regressions

Starting from the relation between the flood maximum discharge and the characteristic elements that generate the flood, which takes the following form:

Qmax=eα0⋅C1α1C2α2.....⋅Cpαpεε					(6)

where: α1 , α2 , …αm  are the regression coefficients, α0  is a constant and ε represents the error of estimation or the residue.

The equation (2) is transformed logarithmically and the estimated value of Qmax, noted Qmax, are the following form:

log {Qmax=α0+α1logC1+α2logC2...αplogCp+ε			(7)

For every section, the computation of the regressions coefficients α0 , α1 , α2 , …αm  was made using about 2/3 of the analysed floods, and the rest were used for validation of the obtained computing relations.

The determination of the regression coefficients was based on minimizing the relation:

i=1nQmax-Qmax2=min					(8)

where: n is the number of events used in computations.

3.1. Checking the pertinence of each independent variables

The checking of the correlation that exists, between the different precipitation characteristics and those of the flood produced’ by them was made by computing the R2 correlation coefficient.

In order to establish the contribution of each Xj independent variable, there were computed the partial correlation coefficients, which express the intensity of the link between the dependent variable one of the independent variables then when it is eliminated the influence of the other independent variables upon them (Duband, 1975).

Taking as example the partial correlation between Y and X1 independent variable, the residual error from the relation of defining the multiple linear regressions takes the following forms:

ξ=Y−(a2X2+a3X3+........+amXm+a0)

(9)

ψ=X1−(b2X2+b3X3+........+bmXm+b0)

a and b coefficients were computed by the least-squares method.

The partial correlation coefficient between Y and X1 variable will take the following form:

rY,X1,X2,...Xm=i=1nξ1iψ1ii=1nξ1i2i=1nψ1i2					(10)

In order to know if an independent variable Xj is or is not significant, in other words, if has a real contribution in estimation of the Y variable, Student test could be made upon the partial correlation coefficients.

Having a dependent variable defined through m independent variables, a θ partial coefficient can be considered, from which there were eliminated the influences of m-1 variables, whose probability density can be expressed as follows [Michel, 1982]:

gn(θ)=1π⋅Γn−m2Γn−m−12⋅(1−θ2)n−m−32				(11)

The transformed variable (n−m−1)⋅θ(1−θ2) follows a Student law with 
(n-m-1) degree of liberty. Therefore, if tp is the value corresponding to the p% probability, there are p chances from 100 to obtain t⋅tp or θ>tp/tp2+n−m−1.

Considering 1% possibility of error, for having only one chance from 100 to mistake, considering that it is a link between Y and X1, X2,….Xm variables, it is necessary that: θ>tt2+n−m−1 with t=0.99.  If (n-m-1) >10, it will be taken as limit value: 1(1+(n−m)26.7⋅(n−m+2)2)

It has been ascertained that, on the whole of the analysed sections, the four characteristics are all significant (table 4).

#### Table 4. Partial correlation coefficients

| River – Gauging station | Independent variables | rY,X1,X2,X3,X4 | Limit value |
| --- | --- | --- | --- |
| ### Valea de Lazuri - VÂRFURILE | hp Dp Pa Qb | 0.876 -0.633 0.257 0.004 | 0.33 |
| Sighişoara - BRAZI | hp Dp Pa Qb | 0.844 -0.820 0.771 -0.629 | 0.301 |
| Sebiş - SEBIS | hp Dp Pa Qb | 0.908 -0.203 0.746 0.585 | 0.301 |
| Valea Roşie - POCOLA | hp Dp Pa Qb | 0.898 -0.426 0.330 0.902 | 0.385 |
| Valea Noua - HUSASAU DE TINCA | hp Dp Pa Qb | 0.970 -0.174 0.373 0.180 | 0.346 |
| Henţ - MORLACA HENT | hp Dp Pa Qb | 0.817 -0.737 0.067 0.665 | 0.256 |
| Iad - LESU AMONTE | hp Dp Pa Qb | 0.969 -0.815 -0.351 0.614 | 0.346 |
| Borod - TOPA DE CRIS | hp Dp Pa Qb | 0.890 -0.552 0.390 0.733 | 0.33 |

3.2. Computation of the regression coefficients

As a result of an optimisation computation there were obtained the values of the regressions coefficients with the following form:

Qmax=eα0⋅Hpα1Dpα2Paα3Qbα4				(12)

The values of these coefficients computed for the analysed 8 sub-basins are presented in table 5.

Table 5. The regression coefficients established

for the computation of the maximum discharge

| River – Gauging station |  | Meteorological station used |  | α0 |  | α1 |  | α2 |  | α3 |  | α4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Valea de Lazuri - VÂRFURILE | GURAHONŢ |  | 0.811 |  | 0.772 |  | -0.26 |  | 0.02 |  | 0.1 |  |
| Sighişoara - BRAZI | GURAHONŢ |  | 0.217 |  | 1.384 |  | -0.910 |  | 0.120 |  | 1.111 |  |
| Sebiş - SEBIŞ | ŞTEI |  | 0.956 |  | 0.707 |  | -0.1 |  | 0.152 |  | 0.433 |  |
| Valea Roşie - POCOLA | HOLOD |  | -0.714 |  | 1.074 |  | -0.2 |  | 0.02 |  | 1.065 |  |
| Valea Nouă - HUSASĂU DE TINCA | HOLOD |  | -1.16 |  | 1.248 |  | -0.16 |  | 0.122 |  | 0.276 |  |
| Henţ - MORLACA HENŢ | VLĂDEASA |  | -0.09 |  | 1.005 |  | -0.4 |  | 0.03 |  | 0.53 |  |
| Iad - LEŞU AMONTE | STÂNA DE VALE |  | 0.007 |  | 1.097 |  | -0.42 |  | 0.04 |  | 0.016 |  |
| Borod - ŢOPA DE CRIŞ | BOROD |  | -1.6 |  | 1.549 |  | -0.38 |  | 0.09 |  | 0.753 |  |

3.3. Checking the relations of computation for the maximum discharge

For emphasizing the fact that on the whole of the analysed floods, the maximum discharges computed by the regression method are close to those observed, it was computed the relative root mean square deviation, at each analysed section, after the following relation:

rRMSE(%)=1ni=1n(Qmaxobs)i−(Qmaxcalc)i(Qmaxobs)i2⋅100			(13)

where: n represents the number of analysed floods.

The figure 2 shows that the relative root mean square error between the computed and observed values is situated in acceptable limits, both for the floods utilized for the computation of the regressions coefficients and for the floods used for validation.

Figure 2. The relative mean square deviation between

the observed and computed maximum discharge

Conclusions

The relations obtained through the method presented above could be used for forecasting the maximum discharges in case of flash floods.

According to the checking made for a part of the analysed floods events, others than those used in computations, the error of estimation of these maximum discharges is small than 20-25%.

In order to use these computation relations in the operative forecasting has been created an automatic computation sheet using the EXCEL program. For the running of the EXCEL sheet it is necessary the daily updating of the mean precipitation on basin, during 10 days before the flood occurring, and the introduction of the precipitation depth hp(mm), the rain duration Dp(h) and the base runoff Qb(m3/s).

Referents

Dauphin J.C. (1992) IDAMS - the Reference Manual, UNESCO

Duband D. (1975) Ridge regression, Note interne, EDF-DTG, France

Georgakakos K. (1996) On the Design of National, Real_time Warning Systems with Capability for Site-Specific, Flash Flood Forecasts  - Iowa Institute of Hydraulic Research

Michel C. (1982) Perfectionnement en Hydrologie - Cemagref Antony

Simota M., Mic R. (1993) Evaluation des seuils critiques de précipitation qui declenchent l'alerte hydrologique à l'interieur d'un systeme informatique d'annonce et de prevision des crues. - Rencontre Hydrologique Franco-Roumaines, Tulcea 1993

*** (1994) Guide to hydrological practices - World Meteorological Organization