- ENVIRONMENTAL EFFECTS OF INDUSTRIAL DREDGING ON ALLUVIAL RIVERBEDS

- László Rákóczi, János Szekeres

- VITUKI, Budapest, Hungary, e-mail: Szekeres@vituki.hu

- Abstract: Several hydropower plants built in Austria and Croatia influence the natural flow and sediment regime of the Croatian-Hungarian reach of River Dráva. The subsequent river channel deformations have recently been increased by gravel and sand excavations in both countries. The bed-load balances of the sub-reaches between four sediment-measuring stations were determined in contrast to the annual dredged volumes. The annual rate of riverbed degradation was estimated using these balances and the shift of discharge rating curves at each station.

- Keywords: riverbed degradation, bed-load balance, dredging, shift of rating curve.

- UMWELTAUSWIRKUNGEN VON INDUSTRIELLEN BAGGERUNGEN IN ALLUVIALEN FLUSSBETTEN

- Zusammenfassung: Das natürliche Abfluss- und Feststoff-Regime der kroatisch-ungarischen Draustrecke wird von den Österreich und Kroatien errichteten Laufkraftwerken beeinflusst.  Die dadurch erzeugten Flussbettveränderungen wurden in letzter Zeit durch die in beiden Staaten durchgeführten Flussbettbaggerungen noch weiter verschärft. Die Bilanz des an den durch vier Feststoffmessstationen erfassten Flussstrecken ermittelten Geschiebes wurde mit dem jährlich gebaggerten Volumen des Flussbettmaterials verglichen. Das jährliche Tempo der Flussbettvertiefung wurde anhand der Geschiebebilanzen sowie der Verschiebungen der Schlüsselkurven der einzelnen Messstationen geschätzt.

- Schlüsselwörter: Flussbettvertiefung; Geschiebebilanzen; Flussbettbaggerung; Verschiebung von Schlüsselkurven.

- 1. Introduction

- The degradation of the river channel in River Dráva has been started several centuries ago and its annual rate is changing according to the intensity of human interventions. This gradual deepening of the riverbed not only directly influences the life conditions of the fluvial flora and fauna but decreases the frequency of floodplain inundations and decreases the ground water level in the riparian regions. The aim of this  paper is  to estimate the annual rate of this process by different methods.

- The natural sediment transporting capacity of River Dráva is considerable, due to its high slopes in its mountainous reach. Even in the lower course, at the confluence with its most important tributary, R. Mura (238 rkm), the water surface slope is about 0.4 m/km. It drops to 0.15 m/km downstream of Bélavár (198,7 rkm) only, where the coarse gravel bed material is gradually refined to coarse sand till Barcs (152.7 rkm). From here, the slope remains almost unchanged over the fine sand-bed reach downstream to Drávaszabolcs (77.7 rkm) (Figure 1).

- River Mura, originally a sediment-laden stream, too, has a slope of about 0,6 m/km upstream of the confluence. Its bed-load contribution to the one of R. Dráva is moderate, due to the river barrages existing in Austria and Croatia. The flow discharge and sediment measuring station is at Letenye, situated 20 km upstream of the confluence. Unfortunately, bed-load sampling has been stopped here in 1980, thus, estimated data on annual bed-load transport can not be supported by actual measurements for the last 25 years.

- 2. Bed-load transport in the investigated reach of River Dráva

- Regular bed-load measurements were started at Barcs and at Drávaszabolcs in 1969, however, the data collection has been discontinued in 1983. This interruption was all the more unfortunate, because in 1982 and 1989 two run-of-river hydropower plants have been constructed on the Croatian reach of River Dráva, upstream of the section dealt with in this paper. Besides, in 1982, both countries started regular dredging activities in various parts of the river with the purpose of improving the conditions for navigation and/or obtaining construction material.

- In 1998, a demand has aroused to continue the sediment data collection parallel with the modernization of measuring methods and equipment. For the sake of this study, two new cross-sections for the sediment and flow discharge measurements were selected and established at Botovo (227.2 rkm) and at Bélavár (198.7 rkm) supplementing the stations Barcs (152.7 rkm) and Drávaszabolcs (77.7 rkm) established in 1961 (Figure 1). The length of the recently investigated reach of Dráva River extends from Botovo to Drávaszabolcs (about 150 km). The details of the bed-load samplers and the tracing method were described elsewhere (Rákóczi and Szekeres 1999).

- 2.1. Bed material samples and their connections to bed-load samples

- Besides some pilot bed material sampling near the two new sediment measuring stations, the local District Water Authority systematically sampled the bed material within the study reach of R. Dráva between 1998 and 2000. The samples were taken at every river km section in 5 points. In Figure 2, showing the distribution of the cross-sectional average grain-sizes along the river, the striking difference of the gravel-bed and sand-bed reaches upstream and downstream of Barcs is clearly visible. Along the Botovo-Barcs reach, several mm changes in grain-sizes can be seen. The reason of it might be the dredging activity, however, the planform of R. Dráva reveals that the finer grains are more characteristic to the fords and the coarser ones to the pools of curvatures of the river channel.

- The influence of the flow regime on the armouring process, characteristic to the surface layer of gravel beds, is demonstrated by change of grain-size composition of the riverbed at Bélavár. Here the average grain diameters varied from 16 to 20 mm in 2000, while in 1998 they were found around 14 mm. In the Botovo region, this effect is enhanced by the daily peak power generation at the nearby Donja Dubrava Croatian hydropower station resulting in repeated sudden water level and flow velocity fluctuations with high riverbed erosion capacity.

- Further examples can be obtained for the bed material - bed-load connection and for the variation of average grain sizes along the cross-sections in Figures 3. and 4. summarizing the results of the sediment measurements. The former shows at Botovo station that at mean flow (467 m3/s) a fully developed armouring could occur on the right- and the left-hand side of the section (2, 3, 4, 5 and 8 sampling points), where the grain sizes are high. The bed-load sampling was not successful at these places, only at the 6. point, where the armouring process was not yet complete. Figure 4. demonstrates that at Bélavár the partial bed-load movement became effective along the entire cross-section at higher flows (983 m3/s) only, when the erosion-protective armour layer had broken up. In the fine-sand bed section of Drávaszabolcs, obviously no resistant armouring could develop, however, at Barcs there were found some signs of armouring on certain places and flow discharges. The reason for several surprisingly low bed-load discharges measured at Barcs even by medium flow discharges could be a temporary lack of sediment supply from the direction of Bélavár due to increased dredging activities over there.

- 2.2. Annual bed-load transport

- The collection of bed-load data in the gravel-bed reaches was hindered by the armouring process on the bed surface and the results influenced by the various sources of error characteristic for the sampling in gravel- and sand-bed river reaches alike (Rákóczi and Szekeres 2002). After repeated checking the measured data and omitting unlikely results, graphical correlations between water and bed-load discharges (bed-load rating curves) could be established for the sediment measuring stations. As an example, the graphical correlations for the data measured at station Barcs is given in Figure 5. Using the multi-annual time series of daily mean flow discharges and the bed-load rating curves, the annual bed-load transport values were calculated for each station. The results are shown in Table 1. For comparison, also the data calculated for Letenye station are given. The reasons for the absence of measured sediment data were described in Chapter 1. By all means, the annual transport values at Letenye are well surpassed by the transport measured at Botovo. It should be considered that several hydropower plants have been constructed also on the River Mura, retarding most part of bed-load arriving from upstream. Besides, the station Letenye is situated 24 km upstream of the confluence with River Dráva, therefore, the unknown bed-load supply from R. Mura to R. Dráva can not be important.

| - Table 1. Annual bed-load transport of Dráva and Mura Rivers (t/yr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Year | - Botovo | - Bélavár | - Barcs | - Dráva-szabolcs | - Letenye |
| - 1986 | - 165 587 | - 56 887 | - 117 776 | - 283 545 | - 52 611 |
| - 1987 | - 126 122 | - 42 059 | - 91 638 | - 228 289 | - 51 937 |
| - 1988 | - 46 849 | - 14 960 | - 35 117 | - 107 287 | - 13 512 |
| - 1989 | - 184 710 | - 65 619 | - 128 513 | - 306 357 | - 176 374 |
| - 1990 | - 52 595 | - 17 389 | - 38 497 | - 96 144 | - 19 727 |
| - 1991 | - 160 236 | - 54 937 | - 114 182 | - 298 318 | - 63 756 |
| - 1992 | - 84 295 | - 27 879 | - 61 607 | - 169 380 | - 38 098 |
| - 1993 | - 144 226 | - 51 819 | - 99 605 | - 231 717 | - 60 910 |
| - 1994 | - 45 052 | - 14 374 | - 33 798 | - 106 131 | - 11 938 |
| - 1995 | - 67 872 | - 22 309 | - 49 867 | - 141 590 | - 23 150 |
| - 1996 | - 178 755 | - 61 605 | - 126 979 | - 322 580 | - 102 050 |
| - 1997 | - 40 936 | - 13 031 | - 30 757 | - 83 811 | - 11 879 |
| - 1998 | - 186 499 | - 66 332 | - 129 645 | - 248 529 | - 23 112 |
| - 1999 | - 121 850 | - 40 571 | - 88 633 | - 241 491 | - 52 042 |
| - 2000 | - 142 105 | - 49 017 | - 100 896 | - 238 799 | - 19 044 |
| - 2001 | - 43 353 | - 13 721 | - 32 684 | - 86 067 | - 2 873 |
| - 2002 | - 62 599 | - 20 927 | - 45 456 | - 108 366 | - 23 962 |
| - 2003 | - 10 358 | - 3 110 | - 8 119 | - 27 448 | - 607 |
| - Average | - 109 038 | - 37 261 | - 77 979 | - 194 024 | - 43 940 |

- Depending on the flow regime within the 1986-2003 period, the total mass of bed-load transport varied between 10 000-186 000 t/year at Botovo, 3 000-66 000 t/year at Bélavár,   8 000-130 000 t/year at Barcs and 27 000-320 000 t/year at Drávaszabolcs. Along the reach between Botovo and Barcs the dynamic bed-load supply depends heavily on the flow regime, the effects of the Croatian hydropower plants and on the volume of annual dredging from the river channel. Here the supply shows a decreasing tendency with the increasing distance from Botovo. The supply increases from Barcs to Drávaszabolcs, mainly due to the erosion of soft banks here.

- 3. Re-supply of dredged bed material in natural ways

- Disregarding the unknown and probably not significant amount of gravel bed-load entering the River Dráva from R. Mura, the gravel supply for the section Botovo-Barcs is not renewable, because the Croatian river barrages hamper the bed-load supply to arrive here. The possible sources of bed-load transport measured at Botovo are the rearrangement of bed material and intermittent selective erosion processes along the 27 km long reach of River Dráva from HPP Donja Dubrava to Botovo. During higher floods when the gates of the barrage are open, some fine gravel might arrive to this reach rolling down toward Botovo, however, this amount is not significant according to the grain-size composition of bed-load samples. Following an occasional beak-up of armoured bed and from bed material loosened by eventual dredging operations, measurable bed-load transport occurs at station Bélavár. The gravel masses of river channel dredging in the Botovo-Bélavár section are not known. They can only be estimated from the difference of average annual bed-load masses at these stations: 109 038 - 37 261= 71 777 t (Table 1).

- The annual dredged bed material masses are given in Table 2. subdivided according to the two riparian countries and to the sand and gravel material. No distinction is made however, regarding the purpose of excavations, e.g. river training or construction industry.  It can be seen that in the period 1986-2003 Hungary and Croatia together excavated 6 899 000 t bed material, 5 560 000 t of which was gravel. The former figure equals 328 000 t/yr, the latter 265 000 t/yr in the average, while the average annual bed-load transport varies from 37 000 t (Bélavár) to 78 000 t (Barcs). These data demonstrate that the bed-load transport balance here is negative, i.e. R. Dráva can not compensate the material dredged from the river channel. The higher transport at Barcs can be explained by the fact that the slope of the river drops remarkably downstream of Bélavár as mentioned in Chapter 1. Consequently, R. Dráva deposits a significant part of its bed load, which is intermittently transported further toward Barcs. Geophysical surveys have shown that the river is able here to re-fill dredged pits in a relatively short time, moreover, to build temporary deposits up to 0.5 m thick over the original bed surface.

| - Table 2. Annually dredged bed material masses in River Dráva (t/yr) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Hungarian dredging |  |  | - Croatian dredging |  |  |  |
| - Year | - sand+ gravel | - gravel 184,2-185,0 rkm | - sand+ gravel 152,7-157,1 rkm | - sand+ gravel | - gravel since 1993 | - since 1993 sand | - Total dredging |
| - 1982 | - 171 000 |  |  | - 472 680 |  |  | - 643 680 |
| - 1983 | - 192 420 |  |  | - 438 300 |  |  | - 630 720 |
| - 1984 | - 238 277 |  |  | - 396 000 |  |  | - 634 277 |
| - 1985 | - 180 000 |  |  | - 316 800 |  |  | - 496 800 |
| - 1986 | - 160 200 |  |  | - 244 534 |  |  | - 404 734 |
| - 1987 | - 206 460 |  |  | - 90 000 |  |  | - 296 460 |
| - 1988 | - 190 800 |  |  | - 180 000 |  |  | - 370 800 |
| - 1989 | - 218 880 |  |  | - 297 000 |  |  | - 515 880 |
| - 1990 | - 116 460 |  |  | - 177 059 |  |  | - 293 519 |
| - 1991 | - 41 904 |  |  | - 109 834 |  |  | - 151 738 |
| - 1992 | - 51 696 |  |  | - 255 861 |  |  | - 307 557 |
| - 1993 | - 60 714 |  |  | - 172 211 | - 130 372 | - 41 839 | - 232 925 |
| - 1994 | - 54 000 |  |  | - 458 860 | - 375 957 | - 82 903 | - 512 860 |
| - 1995 | - 81 104 |  |  | - 428 094 | - 428 094 | - 0 | - 509 198 |
| - 1996 | - 67 000 |  |  | - 279 724 | - 228 352 | - 51 372 | - 346 723 |
| - 1997 | - 64 899 | - 1 890 | - 63 009 | - 470 736 | - 262 181 | - 208 555 | - 535 635 |
| - 1998 | - 115 967 | - 63 844 | - 52 123 | - 558 356 | - 361 800 | - 196 556 | - 674 323 |
| - 1999 | - 175 946 | - 124 733 | - 51 214 | - 560 317 | - 166 059 | - 394 258 | - 736 263 |
| - 2000 | - 201 568 | - 147 254 | - 54 313 | - 238 973 | - 238 435 | - 538 | - 440 541 |
| - 2001 | - 188 041 | - 142 834 | - 45 207 | - 128 833 | - 99 497 | - 29 336 | - 316 874 |
| - 2002 | - 230 771 | - 169 956 | - 60 815 | - 21 775 | - 14 364 | - 7 411 | - 252 545 |
| - Subtotal of marked values |  | - 1 899 729 |  |  | - 3 659 398 |  | - 6 898 576 |

- The river reach Barcs-Drávaszabolcs also shows a negative bed load balance i.e. in contrast to the 78 000 t/yr average annual bed-load transport entering the reach at Barcs, 194 000t/yr is leaving at Drávaszabolcs. This is due to the riverbed and bank erosion along this section. The sand excavation has an average annual value of only 101 000 t/yr. By converting the above data into m3/yr dimension, the average annual bed degradation was obtained as follows:

- Along the 46 km long reach from Bélavár to Barcs, taking a river channel width of 140 m, the degradation is 0.35 cm/yr from the bed-load balance and 2.8 cm/yr due to the dredging;

- Along the 70 km long reach from Barcs to Drávaszabolcs, the two kinds of degradations are 0. 58 cm/yr and 0. 50 cm/yr.

- 4. Checking the rate of riverbed degradations by the changes of hydrological parameters

- Figure 6. shows the characteristic annual flow discharges (maximum, mean and minimum values) for the 1970-2000 period at station Barcs. It can be seen that the trends of the low and mean flows are almost unchanged. The trend of the peak discharges shows a definite decrease presumably caused by the construction of the Croatian hydropower plants. This assumption is supported by the slightly increasing second trendline (dotted) referring to the 1976-2000 period. This analysis suggests that if the annual minimum and mean water levels have a decreasing tendency, the only reason for it can be the degradation of bed level.

- The trends of annual low, mean and high flow gauge heights are to be seen in Figure 7. also for the station Barcs. It can be seen that all the three trends of characteristic water levels definitely decrease due to the lowering of riverbed The same applies also to station Őrtilos (a Hungarian gauge station near Botovo) where the drop of mean flow levels is 1.20 m and at Barcs this value is 1.60 m during 43 years. These data correspond to 2.8 and 3.7 cm/yr respectively. The drop of low flow levels calculated in a similar way is 3.9 cm/yr at Őrtilos and 4.6 cm/yr at Barcs. In case of station Drávaszabolcs the rate of decrease is less, probably due to the backwater effect from River Danube and to the moderate intensity of dredging: 2.4 cm/yr and 3.0 cm/yr.

- The results of these estimations can also be checked by the evaluation of shifting processes of flow discharge rating curves. The upward shift means aggradation and the downward shift the degradation of riverbeds. As an example, the flow discharge rating curves at Barcs is presented in Figure 8. for the period 1972-2000. Between 1972 and 1982 a deepening of about 4.0 cm/yr can be deducted. From that time till 2000 the annual rate of degradation was 3.0 cm/yr in the 400 m3/s discharge range and 2.0 cm/s in the 1000 m3/s range. This means 0.8-1.0 m bed level drop within the investigated 30 years interval.

- At Drávaszabolcs higher rates of bed degradation (5.0-8.0 cm/yr have occurred between 1971-1979, maybe due to the narrowing of the river channel by a series of spur dikes. This has accelerated the deepening process, which later gradually decreased to 1.5-2.0 cm/yr. The latter values are similar to the ones obtained earlier (1.1 cm/yr) by the bed-load budget method.

- 5. Conclusions

- The degradation of the river channel in River Dráva has been started several centuries ago and its annual rate is changing according to the intensity of human interventions. Cutoff of numerous riverbends around the begin of the XXth century decreased the length of the river and increased its slope, i.e. its erosive capacity. Construction of flood protecting dikes along the river and narrowing its width by spur dikes has further increased the celerity of riverbed deepening. Three hydropower plants erected in the Croatian reach since 1975 have drastically changed the flow- and sediment regime of the study reach of R. Dráva. The gravel and sand dredging operations carried out in various sections of the river by Croatia and Hungary since 1986 have accelerated the degradation process. This gradual deepening of the riverbed not only directly influences the life conditions of the fluvial flora and fauna but decreases the frequency of floodplain inundations and decreases the ground water level in the riparian regions. The present paper succeeded to estimate the annual rate of this process by different methods and to separate the effects of the various human interventions. The environmental consequences make it necessary to decrease the dredging activities to a reasonable level in order to let this precious environment with various water habitats to recover and survive.

- 6. References

- Rákóczi, L., Szekeres, J. (2000):  Renewed collection of sediment data on Dráva River.

- XX. Conference of the Danubian Countries, Bratislava, Slovakia. Paper 3.3

- Rákóczi, L., Szekeres, J.  (2002): In-situ checking the performance of bed-load samplers by

- underwater video. Proceedings of the International Conference “River Flow 2002”.

- Louvain la Neuve, Belgium. Vol. 2. pp. 1145-1149. Balkema Publishers