RELATIONSHIPS BETWEEN THE MODULAR VALUES AND VARIATION COEFFICIENTS OF MAXIMUM DISCHARGES IN THE DANUBE RIVER BASIN IN CROATIA

- Danko Biondić1, Darko Barbalić2

Croatian Water, Zagreb, Croatia, e-mail: 1dbiondic@voda.hr;  2darkob@voda.hr

Abstract: This paper presents regional relationships between the modular values and variation coefficients for maximum annual discharges of the 10, 100 and 1.000-year return periods in the Danube River basin in Croatia. Those are calculated on the basis of 90 available sufficiently long homogenous time series of measured discharges. Practical aspects of the relationships use in planning and designing of hydrotechnical systems has also been mentioned.

Keywords: regional relationships, modular values, variation coefficients, maximum discharges, The Danube River basin, Croatia.

VERHALTNISSE ZWISCHEN MODULIWERTEN UND KOEFFIZIENTEN VON VARIATIONEN VON MAXIMALEN FLÜSSEN IM DONAU-FLÜSSGEBIET IN KROATIEN

Zusammenfassung:	Dieses Dokument präsentiert regionale Verhältnisse zwischen Moduliwerten und Koeffizienten von Variationen von maximalen 10, 100 und 1.000-Jahresflüsse im Donau-Fluβgebiet in Kroatien. Diese wurden auf der Basis von 90 vorhandenen, genügend langen homogenen Serien der Durchflüsse bemessenen auf den Fluβwarten kalkuliert. Praktische Aspekte des Gebrauchs von Verhältnisse in Planung und Konstruktion von hydrotechnischen Systemen wurden auch erwähnt.

Schlüsselworte: regionale Verhältnisse, Moduliwerten, Koeffizienten von Variationen, maximale Flüsse, Donau-Fluβgebiet, Kroatien.

1.	Introduction

The Danube River basin is after the Volga River basin the second largest one in Europe with a size of about 817.000 km2, with 18 riparian states and about 82 millions inhabitants. In Croatia it covers approximately 34.000 km2, roughly 60 % of the country’s land area, where approximately 65 % of the total population live. Major Croatian rivers, the Danube, the Sava, the Drava, the Kupa, the Una and the Mura flow through this area. It is located in the Pannonian plain and its rims, with the water divide separating it from the Adriatic catchments running through the Dinaric karst.

A particular socio-economic significance of this area, not only for the Republic of Croatia but also for the greater region emphasizes importance of efficient flood control. Although certain flood control activities in this area date from 19th century, systematic development of flood control systems did not begin until catastrophic floods of the Sava, the Drava and their tributaries during the 1960s. Gradual development of flood control systems in the last four decades has significantly reduced potential damages, a fact proven by successful reduction of numerous recent floods. The most of constructed systems are partially completed, which results in a still existing significant risk of flooding in large areas. Further development of the flood and torrents control systems remains therefore one of the strategic tasks of Croatian water management.

Basic hydrological informations for further planning and designing of flood control systems are maximum discharges of required return periods. Those can be estimated by applying stochastic methods for sufficiently long homogenous time series of measured discharges at gauged locations or by different PUB (Predictions in Ungauged Basins) methods (Hubert, 2002) at locations where time series of measured data are insufficiently long or non-existent. Common hydrological practice also recognizes the use of various methods for evaluation of reliability of such calculated values.

In some cases when time series are insufficiently long for applying standard stochastic methods, maximum discharges of required return periods can be estimated by applying regional relationships between its modular values and variation coefficients of maximum annual discharges. For application of this method, time series of maximum annual discharges should be homogenous, without significant trends and its values of coefficient of variation errors should be sufficiently small.

The aim of this paper is to present these relationships for maximum annual discharges of the 10, 100 and 1.000-year return periods.

Figure 1.	Overview map of the locations of analysed gauging stations in the Danube River basin in Croatia

2.	Theoretical approach and investigation procedure

Regional relationships between the modular values and variation coefficients of maximum annual discharges for the 10, 100 and 1.000-year return periods in the Danube River basin in Croatia are formulated as follows:

QM,RPQM,AV=aRP CVM+bRP								(1)

where:

QM,RP		-	maximum annual discharge for the RP-year return period (m3/s)

QM,AV		-	average maximum annual discharge (m3/s)

CVM		-	variation coefficient of time series of maximum annual discharges

aRP, bRP		-	regional parameters for the RP-year return period.

Subject relationships are defined on the basis of available time series of daily discharges which are stored in the Hydrological data base of Croatian Meteorological and Hydrological Service (Plantić, 1996).

Described theoretical approach can be applied only when the series are sufficiently long (values of its variation coefficient errors should be smaller than 10 %), homogenous and when there are no significant trends. Analysis of time series homogeneity, trends and variation coefficient errors were performed for maximum annual discharge series at all gauging stations in the Danube river basin in Croatia which have observation periods longer or equal than 25 years (105 stations).

The analyses of homogeneity were performed by application of the Wilcoxon‘s test in such a way that the available series was split into two sub-series dependent on times of replacement of hydrometrical equipment on the stations (rods to limnigraphs) and dependent on times of constructions of main hydrotechnical structures with significant impacts on water regimes (reservoirs, main dykes and distribution structures). The presence of trends in time series of maximum annual observed discharges were tested by the Mann’s test. Analysis of coefficient of variation errors were performed by Kritzky-Menkel‘s:

σCVM=±CVM2n−1⋅1+3CVM2<0,10	(10 %)					(2)

and UNESCO‘s tests (Žugaj, 1998):

σCVM=±CVM1+2CVM22n<0,10	(10 %),						(3)

where:

σCVM	-	variation coefficient error

CVM	-	variation coefficient of time series of maximum annual discharges

n	-	number of years.

On the basis of performed analysis, 90 time series of maximum annual discharges were selected for further analysis.

The next step was probabilistic analysis of selected time series of maximum annual discharges. Maximum annual discharges of 10-years return period were calculated by application of empirical distribution, and maximum annual discharges of 100 and 1.000-year return periods were calculated by applications of Normal distribution (2 stations), Log-Normal distribution (24 stations), Gamma two-parameters distribution (7 stations), Gumbel distribution (10 stations), Pearson III distribution (23 stations) and Log-Pearson III distribution (24 stations). The goodness of fit was tested by application of the Kolmogoroff-Smirnoff test.

The last step was regression analysis of the modular values and variation coefficients of maximum annual discharges. Correlation coefficients (r) of calculated linear relationships amount 0,81 for 10-year return period and 0,92 for 100 and 1.000-year return period. Confidence intervals for 95 % probability (σ) amount ±15,1 % for 10-year return period, ±15,3 % for 100-year return period and ±28,4 % for 1.000-year return period.

Table 1.	Basic characteristics of analysed gauging stations in the Danube River basin in Croatia

| No. (Fig.1) | RIVER | GAUGING STATION | OBSERVING PERIOD (NUMBER OF YEARS) | DISCHARGES (m3/s) |  | VARIATION COEFFICIENT OF TIME SERIES OF MAXIMUM ANNUAL DISCHARGES | MODULAR VALUES OF MAXIMUM DISCHARGES FOR RETURN PERIODS (years) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MEAN | AVERAGE MAXIMUM ANNUAL |  | 10 | 100 | 1000 |
| THE DANUBE RIVER |  |  |  |  |  |  |  |  |  |
| 1 | DUNAV | BATINA | 1951. - 1989. (39) | 2313 | 4841 | 0.228 | 1.31 | 1.62 | 1.92 |
| 2 | DUNAV | ERDUT | 1950. - 1989. (40) | 2852 | 5443 | 0.242 | 1.34 | 1.76 | 2.20 |
| THE DRAVA RIVER BASIN |  |  |  |  |  |  |  |  |  |
| 3 | DRAVA | VARAŽDIN | 1951. - 1981. (31) | 341 | 1286 | 0.350 | 1.41 | 2.16 | 2.81 |
| 4 | DRAVA | BOTOVO | 1961. - 1998. (38) | 517 | 1596 | 0.297 | 1.48 | 1.93 | 2.47 |
| 5 | DRAVA | TEREZINO POLJE | 1961. - 1998. (38) | 526 | 1506 | 0.327 | 1.58 | 2.12 | 2.93 |
| 6 | DRAVA | DONJI MIHOLJAC | 1926. - 1998. (71) | 543 | 1360 | 0.232 | 1.29 | 1.67 | 1.99 |
| 7 | DRAVA | BELIŠĆE | 1962. - 1993. (31) | 556 | 1405 | 0.265 | 1.44 | 1.83 | 2.31 |
| 8 | MURA | MURSKO SREDIŠČE | 1926. - 1998. (67) | 171 | 732 | 0.390 | 1.60 | 2.31 | 3.11 |
| 9 | MURA | GORIČAN | 1926. - 1998. (70) | 161 | 642 | 0.395 | 1.55 | 2.14 | 2.67 |
| 10 | BEDNJA | ŽELJEZNICA | 1959. - 1998. (40) | 4.04 | 59.9 | 0.409 | 1.60 | 2.55 | 3.57 |
| 11 | BEDNJA | TUHOVEC | 1958. - 1998. (36) | 6.46 | 80.9 | 0.341 | 1.39 | 1.70 | 1.80 |
| 12 | BEDNJA | LUDBREG | 1947. - 1998. (52) | 7.29 | 80 | 0.411 | 1.60 | 2.15 | 2.65 |
| 14 | GLIBOKI POTOK | MLAČINE | 1970. - 1998. (29) | 0.747 | 19.2 | 0.385 | 1.56 | 1.89 | 2.18 |
| 15 | KOPRIVNICA | KOPRIVNICA | 1951. - 1998. (46) | 0.645 | 23.5 | 0.551 | 1.93 | 3.07 | 4.51 |
| 17 | VOĆINKA | MIKLEUŠ | 1960. - 1998. (39) | 2.18 | 54.5 | 0.460 | 1.71 | 2.07 | 2.27 |
| THE SAVA RIVER BASIN |  |  |  |  |  |  |  |  |  |
| 19 | SAVA | JESENICE | 1964. - 1995. (32) | 276 | 1846 | 0.341 | 1.45 | 2.03 | 2.54 |
| 20 | SAVA | PODSUSED | 1949. - 1995. (47) | 306 | 1738 | 0.295 | 1.48 | 2.01 | 2.73 |
| 21 | SAVA | ZAGREB | 1926. - 1995. (70) | 314 | 1775 | 0.266 | 1.32 | 1.73 | 2.05 |
| 22 | SAVA | RUGVICA | 1926. - 1995. (67) | 312 | 1502 | 0.246 | 1.38 | 1.77 | 2.21 |
| 23 | SAVA | CRNAC | 1955. - 1992. (38) | 529 | 1935 | 0.101 | 1.11 | 1.22 | 1.28 |
| 24 | SAVA | JASENOVAC | 1926. - 1991. (64) | 784 | 1977 | 0.112 | 1.16 | 1.35 | 1.54 |
| 25 | SAVA | STARA GRADIŠKA | 1937. - 1991. (54) | 788 | 1899 | 0.127 | 1.16 | 1.34 | 1.48 |
| 26 | SAVA | MAČKOVAC | 1951. - 1990. (40) | 823 | 2115 | 0.153 | 1.19 | 1.43 | 1.63 |
| 27 | SAVA | DAVOR | 1958. - 1993. (34) | 931 | 2321 | 0.132 | 1.17 | 1.37 | 1.53 |
| 28 | SAVA | SLAVONSKI KOBAŠ | 1926. - 1993. (65) | 974 | 2411 | 0.145 | 1.23 | 1.40 | 1.58 |
| 29 | SAVA | SLAVONSKI BROD | 1945. - 1993. (49) | 944 | 2466 | 0.148 | 1.17 | 1.38 | 1.52 |
| 30 | SAVA | ŽUPANJA | 1929. - 1998. (65) | 1159 | 2942 | 0.163 | 1.28 | 1.49 | 1.76 |
| 31 | BREGANA | BREGANA REMONT | 1970. - 1998. (29) | 1.38 | 21.5 | 0.295 | 1.45 | 1.92 | 2.41 |
| 32 | LIPOVAČKA GRADNA | HAMOR | 1948. - 1998. (50) | 0.381 | 4.26 | 0.322 | 1.42 | 2.05 | 2.65 |
| 33 | RUDARSKA GRADNA | RUDARSKA DRAGA | 1957. - 1994. (38) | 0.255 | 4.49 | 0.393 | 1.53 | 2.31 | 3.11 |
| 35 | VRAPČAK | ZAGREB | 1961. - 1998. (38) | 0.168 | 4.73 | 0.585 | 1.51 | 2.83 | 4.16 |
| 42 | SUNJA | SUNJA | 1965. - 1992. (32) | 2.82 | 87.4 | 0.370 | 1.53 | 1.99 | 2.33 |
| 43 | ŠUMETLICA | CERNIK | 1972. - 1998. (27) | 0.291 | 6.45 | 0.421 | 1.52 | 2.32 | 3.08 |
| 44 | SUTLA | BREZNO | 1946. - 1975. (30) | 1.34 | 21.8 | 0.277 | 1.35 | 1.85 | 2.28 |
| 45 | SUTLA | MILJANA | 1947. - 1976. (30) | 4.19 | 65.8 | 0.062 | 1.08 | 1.16 | 1.22 |
| 46 | SUTLA | ZELENJAK | 1958. - 1998. (41) | 7.27 | 123 | 0.323 | 1.42 | 2.00 | 2.52 |
| 48 | KRAPINA | KUPLJENOVO | 1964. - 1998. (35) | 11.8 | 154 | 0.278 | 1.34 | 1.78 | 2.13 |
| 49 | ZELINA | BOŽJAKOVINA | 1957. - 1998. (38) | 1.64 | 28.9 | 0.362 | 1.55 | 2.24 | 2.99 |
| 50 | LONJA | BISAG | 1952. - 1982. (31) | 0.768 | 15.6 | 0.190 | 1.21 | 1.44 | 1.59 |
| 51 | LONJA | LONJICA | 1972. - 1998. (27) | 1.88 | 21.8 | 0.393 | 1.27 | 2.23 | 2.94 |
| 52 | ČESMA | NARTA | 1958. - 1998. (41) | 5.43 | 51.1 | 0.323 | 1.38 | 1.95 | 2.41 |
| 53 | ČESMA | ČAZMA | 1963. - 1998. (36) | 15.1 | 98.3 | 0.314 | 1.50 | 2.01 | 2.60 |
| 54 | ILOVA | VELIKO VUKOVJE | 1945. - 1998. (52) | 7.36 | 71.5 | 0.336 | 1.25 | 1.94 | 2.36 |
| 55 | BIJELA | BADLJEVINA | 1949. - 1975. (27) | 1.53 | 19.4 | 0.257 | 1.29 | 1.47 | 1.56 |
| 56 | ORLJAVA | PLETERNICA | 1946. - 1998. (53) | 5.22 | 60.7 | 0.414 | 1.53 | 1.96 | 2.28 |
| 58 | UNA | HRVATSKA KOSTAJNICA | 1926. - 1991. (65) | 228 | 1138 | 0.209 | 1.22 | 1.47 | 1.60 |
| THE KUPA RIVER BASIN |  |  |  |  |  |  |  |  |  |
| 59 | KUPA | KUPARI | 1951. - 1998. (48) | 13.5 | 141 | 0.161 | 1.21 | 1.43 | 1.60 |
| 60 | KUPA | HRVATSKO | 1957. - 1998. (40) | 20.5 | 287 | 0.221 | 1.34 | 1.65 | 1.96 |
| 61 | KUPA | PETRINA | 1951. - 1992. (42) | 26.6 | 455 | 0.339 | 1.41 | 1.97 | 2.51 |
| 62 | KUPA | RADENCI | 1951. - 1992. (42) | 53.7 | 647 | 0.209 | 1.25 | 1.46 | 1.60 |
| 63 | KUPA | PRIBANJCI | 1949. - 1985. (37) | 61.8 | 675 | 0.203 | 1.28 | 1.58 | 1.85 |
| 64 | KUPA | LADEŠIC DRAGA | 1956. - 1998. (42) | 58.4 | 700 | 0.187 | 1.22 | 1.39 | 1.50 |
| 65 | KUPA | KAMANJE | 1957. - 1998. (40) | 73.3 | 811 | 0.158 | 1.18 | 1.40 | 1.56 |
| 66 | KUPA | BRODARCI | 1957. - 1998. (41) | 110 | 944 | 0.162 | 1.20 | 1.38 | 1.50 |
| 67 | KUPA | REČICA | 1948. - 1982. (35) | 171 | 1137 | 0.236 | 1.28 | 1.47 | 1.58 |
| 68 | KUPA | JAMNIČKA KISELICA | 1948. - 1978. (31) | 180 | 967 | 0.256 | 1.46 | 1.85 | 2.43 |
| 69 | KUPA | ŠIŠINEC | 1950. - 1991. (41) | 182 | 949 | 0.148 | 1.17 | 1.41 | 1.58 |
| 70 | KUPA | FARKAŠIĆ | 1965. - 1992. (26) | 196 | 1048 | 0.195 | 1.22 | 1.52 | 1.75 |
| 71 | KUPA | BREST | 1926. - 1974. (49) | 206 | 1006 | 0.170 | 1.21 | 1.37 | 1.47 |
| 72 | CABRANKA | ZAMOST | 1950. - 1998. (49) | 3.72 | 76 | 0.341 | 1.51 | 2.15 | 2.82 |
| 73 | KUPICA | BROD NA KUPI | 1951. - 1998. (48) | 13.7 | 146 | 0.322 | 1.33 | 2.01 | 2.58 |
| 74 | KUPČINA | STRMAC | 1959. - 1998. (40) | 2.18 | 22.7 | 0.403 | 1.75 | 2.53 | 3.88 |
| 75 | KUPČINA | LAZINA BRANA | 1973. - 1998. (26) | 2.07 | 22.9 | 0.188 | 1.21 | 1.35 | 1.41 |
| 76 | KRIŽ POTOK | C.P. KRIŽ | 1963. - 1998. (36) | 0.312 | 14.3 | 0.371 | 1.57 | 2.29 | 3.21 |
| 77 | VELA VODA | CRNI LUG | 1963. - 1998. (36) | 0.208 | 7.98 | 0.326 | 1.50 | 2.10 | 2.81 |
| 78 | BELA VODA | CRNI LUG | 1963. - 1998. (36) | 0.106 | 5.42 | 0.389 | 1.68 | 2.49 | 3.44 |
| 79 | LESKA | LESKA | 1963. - 1998. (26) | 0.015 | 0.603 | 0.414 | 1.55 | 2.39 | 3.19 |
| 81 | GORNJA DOBRA | LUKE | 1947. - 1998. (51) | 7.04 | 109 | 0.207 | 1.22 | 1.54 | 1.76 |
| 82 | GORNJA DOBRA | OGULINSKI HRELJIN | 1948. - 1975. (28) | 4.65 | 52.3 | 0.129 | 1.15 | 1.35 | 1.49 |
| 83 | GORNJA DOBRA | TURKOVIĆI | 1963. - 1998. (33) | 10 | 118 | 0.180 | 1.25 | 1.51 | 1.74 |
| 84 | VITUNJČICA | BRESTOVAC | 1948. - 1973. (25) | 3.44 | 28.9 | 0.173 | 1.27 | 1.49 | 1.73 |
| 85 | DONJA DOBRA | TROŠMARIJA | 1960. - 1998. (39) | 27.9 | 163 | 0.164 | 1.19 | 1.44 | 1.64 |
| 87 | DONJA DOBRA | DONJE STATIVE | 1960. - 1998. (39) | 34.9 | 249 | 0.219 | 1.24 | 1.58 | 1.81 |
| 88 | RIBNJAK | LUČANJEK | 1948. - 1975. (27) | 3.01 | 16.4 | 0.261 | 1.39 | 1.82 | 2.29 |
| 89 | GLOBORNICA | GENERALSKI STOL | 1949. - 1975. (27) | 1.02 | 18.8 | 0.350 | 1.60 | 2.26 | 3.22 |
| 90 | MREŽNICA | JUZBAŠIĆI | 1947. - 1998. (46) | 12.4 | 92.4 | 0.260 | 1.32 | 1.70 | 2.00 |
| 91 | MREŽNICA | MRZLO POLJE | 1947. - 1998. (52) | 29.5 | 254 | 0.204 | 1.29 | 1.54 | 1.75 |
| 92 | TOUNJČICA | OŽANIĆI | 1948. - 1975. (28) | 10.4 | 70.1 | 0.176 | 1.24 | 1.55 | 1.87 |
| 93 | MUNJAVČICA | JOSIPDOL | 1948. - 1975. (27) | 0.431 | 3 | 0.380 | 1.53 | 2.26 | 3.02 |
| 94 | KORANA | KORANA | 1952. - 1986. (35) | 3.09 | 19.2 | 0.280 | 1.46 | 1.95 | 2.63 |
| 96 | KORANA | SLUNJ | 1964. - 1998. (29) | 11.1 | 137 | 0.353 | 1.59 | 2.17 | 2.94 |
| 97 | KORANA | VELJUN | 1949. - 1998. (44) | 23 | 264 | 0.283 | 1.37 | 1.75 | 2.05 |
| 98 | KORANA | VELEMERIĆ | 1946. - 1997. (46) | 28.8 | 327 | 0.285 | 1.38 | 1.88 | 2.35 |
| 99 | PLITVIČKA JEZERA - KOZJAK | KOZJAK MOST | 1953. - 1991. (33) | 3.55 | 16.2 | 0.361 | 1.50 | 2.05 | 2.47 |
| 100 | PLITVIČKA JEZERA - MATICA | PLITVIČKI LJESKOVAC | 1952. - 1991. (37) | 2.42 | 12 | 0.555 | 1.14 | 1.95 | 2.48 |
| 101 | SLUNJČICA | SLUNJ | 1949. - 1983. (35) | 9.48 | 58.5 | 0.171 | 1.24 | 1.38 | 1.48 |
| 102 | RADONJA | TUŠILOVIĆ | 1949. - 1973. (25) | 3.55 | 24.4 | 0.227 | 1.36 | 1.67 | 1.99 |
| 103 | GLINA | MALJEVAC | 1953. - 1986. (34) | 3.37 | 38.5 | 0.362 | 1.34 | 2.14 | 2.79 |
| 104 | GLINA | VRANOVINA | 1947. - 1998. (45) | 14.1 | 146 | 0.498 | 1.88 | 2.81 | 4.24 |
| 105 | GLINA | GLINA | 1952. - 1998. (40) | 18.3 | 178 | 0.445 | 1.63 | 2.46 | 3.41 |

3.	Results

By applying the described methodology, regional relationships between the modular values and variation coefficients of maximum annual discharges for the 10, 100 and 1.000-year return periods in the Danube River basin in Croatia were defined. The obtained results are shown in figures 2, 3, 4 and 5.

Figure 2.	Regional relationship between the modular values and variation coefficients of maximum annual discharges for the 10-year return periods in the Danube River basin in Croatia

Figure 3.	Regional relationship between the modular values and variation coefficients of maximum annual discharges for the 100-year return periods in the Danube River basin in Croatia

Figure 4.	Regional relationship between the modular values and variation coefficients of maximum annual discharges for the 1.000-year return periods in the Danube River basin in Croatia

Figure 5.	Comparison of calculated regional relationships between the modular values and variation coefficients of maximum annual discharges in the Danube River basin in Croatia

Table 2.	Summary overview on calculated parameters of regional relationships

| # DISCHARGES | # PARAMETERS OF REGIONAL RELATIONSHIPS |  |  |  |
| --- | --- | --- | --- | --- |
|  | aRP | bRP | r | σ(%) |
| MAXIMUM ANNUAL FOR THE 10-YEARS RETURN PERIOD | 1,29 | 1,015 | 0,81 | 15,1 |
| MAXIMUM ANNUAL FOR THE 100-YEARS RETURN PERIOD | 3,35 | 0.891 | 0,92 | 15,3 |
| MAXIMUM ANNUAL FOR THE 1.000-YEARS RETURN PERIOD | 5,69 | 0.684 | 0,92 | 28,4 |

Comparison of subject and some previously published relationships was performed and results are shown in figures 6, 7 and 8.

Figure 6.	Comparison of regional relationship between the modular values and variation coefficients of maximum annual discharges for the 10-year return periods in the Danube River basin in Croatia with previously published relationships

Figure 7.	Comparison of regional relationship between the modular values and variation coefficients of maximum annual discharges for the 100-year return periods in the Danube River basin in Croatia with previously published relationships

Figure 8.	Comparison of regional relationship between the modular values and variation coefficients of maximum annual discharges for the 1.000-year return periods in the Danube River basin in Croatia with previously published relationships

4.	Conclusion

The relationships presented in this paper were calculated on the basis of 90 available sufficiently long homogenous time series of observed discharges on gauging stations in the Danube River basin in Croatia. Because of high correlation coefficients (r) and confidence intervals (σ) of about ±15 %, relationships for 10 and 100-year return periods can be used in hydrological practice for estimations of maximum discharges in the cases when time series of measured discharges are insufficiently long for applying standard stochastic methods. Calculated relationship for 1.000-year return period has also high correlation coefficient (r = 0,92), but it is less reliable because of wider confidence interval (σ = ±28,4 %).

Presented relationships can be used for estimations of peak discharges on the basis of average maximum annual discharges, and variation coefficients of time series of maximum annual discharges. It is required that used time series of annual maximum discharges are homogenous and without significant trend, as well as that errors of their variation coefficients in accordance to Kritzky-Menkel‘s and UNESCO‘s tests are less than 10 %. For application of subject relationships, it is necessery to take into account their confidence intervals.

Calculated relationships for the Danube River basin in Croatia are very similar with previously published relationships for Croatian karst including Adriatic basins (Žugaj, 1995), and therefore can be applied for the whole Croatian territory.

5.	References

Hubert, P. (2002): IAHS Decade on Prediction in Ungauged Basins (PUB). Proceedings of

the XXIth Conference of the Danube Countries on Hydrological Forecasting and Hydrological Bases of Water Management, Bucharest, Romania.

Jovanović, S., Radić, Z. (1985): Regional Assessment of Hydrological Parameters

(Regionalna analiza hidroloških parametara). Vodoprivreda, 93, 3-12, JDON, Belgrade, Serbia.

Plantić, K. (1996): Hydrological Data Base for the Danube Catchment Area in Croatia.

Proceedings of the XVIIth Conference of the Danube Countries on Hydrological Forecasting and Hydrological Bases of Water Management, Graz, Austria.

Regional Zusammenarbeit der Donäulander (1986): Die Donau und ihr Einzugsgebeit, Eine

hydrologische Monographie. Bayerisches Landesamt für Wasserwirtschaft, Munich, Germany.

Žugaj, R. (1995): Regional Hydrological Analysis in Croatian Karst (Regionalna hidrološka

analiza u kršu Hrvatske). HHD, Zagreb, Croatia.

Žugaj, R. (1998): Lengths of Flow Series of Drava Catchment Area in Croatia. Proceedings

of the XIXth Conference of the Danube Countries on Hydrological Forecasting and Hydrological Bases of Water Management, Osijek, Croatia.